1. A door sensor is connected to bit 1 of port B and LED is connected to bit 7 of port C.  Monitor the door sensor, and when it opens, turn the light

Program

#include <avr/io.h> // standard AVR header

int main(void)

{

DDRB = DDRB & 0b11111101; // pin 1 of Port B is input

DDRC = DDRC | 0b10000000; // pin 7 of Port C is output

while (1)

{

if (PINB & 0b00000010) // check pin 1 (2nd pin) of PINB

PORTC = PORTC | 0b10000000; // set pin 7 (8th pin) of PORTC

else

PORTC = PORTC & 0b01111111; // clear pin 7 (8th pin) of PORTC

}

return 0;

}

Circuit Configuration

2. 7-Segment Display

Program

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

unsigned char seg_code[10] = {

0x3F, // 0

0x06, // 1

0x5B, // 2

0x4F, // 3

0x66, // 4

0x6D, // 5

0x7D, // 6

0x07, // 7

0x7F, // 8

0x6F // 9

};

int main(void)

{

DDRB = 0xFF; // PORTB as output

while(1)

{

for(int i = 0; i < 10; i++)

{

PORTB = seg_code[i];

_delay_ms(100);

}

}

}

Circuit Configuration

3. Temperature Sensors

Use the folder

4. LCD with ABCD display

Program

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

/* LCD Control Pins */

#define LCD_CTRL PORTC

#define LCD_CTRL_DDR DDRC

#define RS PC0

#define RW PC1

#define EN PC2

/* LCD Data Port */

#define LCD_DATA PORTB

#define LCD_DATA_DDR DDRB

void lcd_cmd(unsigned char cmd)

{

LCD_DATA = cmd;

LCD_CTRL &= ~(1 << RS); // RS = 0

LCD_CTRL &= ~(1 << RW); // RW = 0

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_data(unsigned char data)

{

LCD_DATA = data;

LCD_CTRL |= (1 << RS); // RS = 1

LCD_CTRL &= ~(1 << RW); // RW = 0

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_init()

{

LCD_DATA_DDR = 0xFF;

LCD_CTRL_DDR |= (1<<RS)|(1<<RW)|(1<<EN);

_delay_ms(20);

lcd_cmd(0x38); // 8-bit, 2 line

lcd_cmd(0x0C); // Display ON

lcd_cmd(0x06); // Auto increment

lcd_cmd(0x01); // Clear display

_delay_ms(2);

}

int main(void)

{

lcd_init();

lcd_data('A');

_delay_ms(100);

lcd_data('B');

_delay_ms(100);

lcd_data('C');

_delay_ms(100);

lcd_data('D');

_delay_ms(100);

while (1);

}

Circuit Diagram

5. Your Name on LCD

Program

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

/* LCD Control Pins */

#define LCD_CTRL PORTC

#define LCD_CTRL_DDR DDRC

#define RS PC0

#define RW PC1

#define EN PC2

/* LCD Data Port */

#define LCD_DATA PORTB

#define LCD_DATA_DDR DDRB

void lcd_cmd(unsigned char cmd)

{

LCD_DATA = cmd;

LCD_CTRL &= ~(1 << RS); // RS = 0

LCD_CTRL &= ~(1 << RW); // RW = 0

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_data(unsigned char data)

{

LCD_DATA = data;

LCD_CTRL |= (1 << RS); // RS = 1

LCD_CTRL &= ~(1 << RW); // RW = 0

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_string(char *str)

{

while (*str)

{

lcd_data(*str);

str++;

}

}

void lcd_init()

{

LCD_DATA_DDR = 0xFF;

LCD_CTRL_DDR |= (1<<RS)|(1<<RW)|(1<<EN);

_delay_ms(20);

lcd_cmd(0x38); // 8-bit, 2 line, 5x7

lcd_cmd(0x0C); // Display ON, cursor OFF lcd_cmd(0x06); // Increment cursor

lcd_cmd(0x01); // Clear display

_delay_ms(2);

}

int main(void)

{

lcd_init();

lcd_cmd(0x80); // First line

lcd_string("SIMANTA");

lcd_cmd(0xC0); // Second line

lcd_string("KASAJU");

while (1);

}

6. Counter

Program

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdlib.h> // for itoa

/* LCD Control Pins */

#define LCD_CTRL PORTC

#define LCD_CTRL_DDR DDRC

#define RS PC0

#define RW PC1

#define EN PC2

/* LCD Data Pins */

#define LCD_DATA PORTB

#define LCD_DATA_DDR DDRB

/* LCD Functions */

void lcd_cmd(unsigned char cmd)

{

LCD_DATA = cmd;

LCD_CTRL &= ~(1 << RS);

LCD_CTRL &= ~(1 << RW);

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_data(unsigned char data)

{

LCD_DATA = data;

LCD_CTRL |= (1 << RS);

LCD_CTRL &= ~(1 << RW);

LCD_CTRL |= (1 << EN);

_delay_ms(2);

LCD_CTRL &= ~(1 << EN);

}

void lcd_string(char *str)

{

while (*str)

lcd_data(*str++);

}

void lcd_init()

{

LCD_DATA_DDR = 0xFF;

LCD_CTRL_DDR |= (1 << RS) | (1 << RW) | (1 << EN);

_delay_ms(20);

lcd_cmd(0x38); // 8-bit, 2 line

lcd_cmd(0x0C); // Display ON

lcd_cmd(0x06); // Auto increment

lcd_cmd(0x01); // Clear LCD

}

/* Timer1 1-second delay */

void timer1_delay_1s()

{

TCNT1 = 49911; // preload for 1 second TCCR1A = 0x00;

TCCR1B = (1 << CS12) | (1 << CS10); // Prescaler 1024

while (!(TIFR & (1 << TOV1))); // Wait for overflow

TIFR |= (1 << TOV1); // Clear overflow flag

}

int main(void)

{

char buffer[6];

int count = 0;

lcd_init();

lcd_cmd(0x80);

lcd_string("Counter:");

while (1)

{

lcd_cmd(0xC0); // Second line

itoa(count, buffer, 10); // int to string

lcd_string(buffer);

count++;

timer1_delay_1s();

lcd_cmd(0x01); // Clear LCD

lcd_cmd(0x80);

lcd_string("Counter:");

}

} Circuit