- Write an assembly level program to print a given string.

Explanation:

- Create a string

- Load the effective address of the string in dx using LEA command

- Print the string by calling the interrupt with 9H in AH

- The string must be terminated by ‘$’ sign

.MODEL SMALL

.STACK 100H

.DATA

;The string to be printed

STRING DB 'This is a sample string', '$'

.CODE

MAIN PROC FAR

MOV AX,@DATA

MOV DS,AX

; load address of the string

LEA DX,STRING

;output the string

;loaded in dx

MOV AH,09H

INT 21H

;interrupt to exit

MOV AH,4CH

INT 21H

MAIN ENDP

END MAIN

- Write ALP to read string, display each word in new line in cleared screen, count no of words and display the count.

.MODEL SMALL

.STACK 100

.DATA

STRING LABEL BYTE

MAXLEN DB 60

ACTLEN DB ?

STR DB 60 DUP(?)

.CODE

MAIN PROC FAR

MOV AX, @DATA

MOV DS, AX

MOV ES, AX

LEA DX, STRING

MOV AH, 0AH

INT 21H

CALL CLR_SCR

MOV CL, ACTLEN

MOV CH, 00H

MOV BX, 00H

MOV AL, 00H

NEXT: MOV DL, STR[BX]

CMP DL, ‘ ’

JE NEW

MOV AH, 02H

INT 21H

JMP SKIP

NEW: INC EL

CALL NEW_L

SKIP: INC BX

LOOP NEXT

MOV DL, EL

MOV AH, 02H

INT 21H

MOV AX, 4C00H

INT 21H

MAIN ENDP

CLR_SCR PROC NEAR

MOV AH, 0600H

MOV BH, 07H

MOV CX, 0000H

MOV DX, 184FH

INT 10H

RET

CLR_SCR ENDP

NEW_L PROC NEAR

MOV AH, 0601H

MOV BH, 07H

MOV CX, 0000H

MOV DX, 184FH

INT 10H

RET

NEW_L ENDP

END MAIN

- Write ALP to read string and count number of vowels, consonants, numerals and other characters and display the count.

.MODEL SMALL

.STACK 100

.DATA

STRING LABEL BYTE

MAXLEN DB 60

ACTLEN DB ?

STR DB 60 DUP(?)

VOWEL DB 0

CONSONANT DB 0

NUMERAL DB 0

OTHER DB 0

.CODE

MAIN PROC FAR

MOV AX, @DATA

MOV DS, AX

MOV ES, AX

LEA DX, STRING

MOV AH, 0AH

INT 21H

MOV EL, 00H

MOV CL, ACTLEN

MOV CH, 00H

MOV BX, 00H

NEXT: MOV DL, STR[BX]

CMP DL, ‘0’

JB N1

CMP DL, ‘9’

JA N1

MOV EL, NUMERAL

INC EL

MOV NUMERAL, EL

JMP N2

N1: CMP DL, ‘A’

JB OTH

CMP DL, ‘Z’

JB ALP

CMP DL, ‘a’

JB OTH

CMP DL, ‘z’

JA OTH

ALP: CMP DL, ‘A’

JE VOW

CMP DL, ‘a’

JE VOW

CMP DL, ‘E’

JE VOW

CMP DL, ‘e’

JE VOW

CMP DL, ‘I’

JE VOW

CMP DL, ‘i’

JE VOW

CMP DL, ‘O’

JE VOW

CMP DL, ‘o’

JE VOW

CMP DL, ‘U’

JE VOW

CMP DL, ‘u’

JE VOW

MOV EL, CONSONANT

INC EL

MOV CONSONANT, EL

JMP N2

VOW: MOV EL, VOWEL

INC EL

MOV VOWEL, EL

JMP N2

OTH: MOV EL, OTHER

INC EL

MOV OTHER, EL

N2: LOOP NEXT

MOV DL, VOWEL

MOV AH, 02H

INT 21H

MOV DL, CONSONANT

MOV AH, 02H

INT 21H

MOV DL, NUMERALS

MOV AH, 02H

INT 21H

MOV DL, OTHER

MOV AH, 02H

INT 21H

MOV AX, 4C00H

INT 21H

MAIN ENDP

END MAIN

- Write ALP to sort array of ten numbers stored in memory. Display after sorting.

.MODEL SMALL

.STACK 64

.DATA

ARR 11H, 20H, 9H, 5H, 16H, 22H, 12H, 14H, 8H, 17H

.CODE

MAIN PROC FAR

MOV AX, @DATA

MOV DS, AX

LEA DI, ARR

MOV CX, 09H

NEXT: CALL PERFORM

LOOP NEXT

LEA DI, ARR

MOV DL, [ARR]

MOV AH, 02H

INT 21H

MOV AX, 4C00H

INT 21H

MAIN ENDP

PERFORM PROC NEAR

PUSH CX

COM: MOV CX, 00H

MOV AL, [DI + CX]

MOV BL, [DI + CX + 1]

CMP AL, BL

JNC SKIP

PUSH AL

PUSH BL

POP AL

POP BL

MOV [DI + CX], AL

MOV [DI + CX + 1], BL

SKIP: INC CX

CMP CX, 0AH

JNE COM

POP CX

RET

PERFORM ENDP

END MAIN

- To write an assembly language program to reverse the given string.

ASSUME CS : CODE, DS : DATA

CODE SEGMENT

MOV AX, DATA

MOV DS, AX

MOV CL, COUNT

MOV SI, OFFSET STR1

MOV DI, COUNT - 1

BACK:MOV AL, [SI]

XCHG [DI], AL

MOV [SI], AL

INC SI

DEC DI

DEC CL

JNZ BACK

HLT

CODE ENDS

DATA SEGMENT

STR1 DB ‘MPMC$’

COUNT EQU 04H

STR2 DB DUP (0)

DATA ENDS

END