# NEPAL COLLEGE OF INFORMATION TECHNOLOGY

Balkumari, Lalitpur

Affiliated to Pokhara University

# LAB REPORT FOR EMBEDDED SYSTEMS

LAB REPORT 9: VHDL CODE FOR FLIP FLOP

Submitted by:	Submitted to:

Name: Arpan Adhikari	Er. Simanta Kasaju Roll No.: 231309

BE-CE (5th SEM)

| S. N | LAB TOPIC | DATE | REMARKS |
| --- | --- | --- | --- |
| 1. | VHDL Code (OR & AND Gate) | 20th Nov, 2025 |  |
| 2. | VHDL Code (OR & NOT Gate) using testbench | 23rd Nov |  |
| 3. | Universal Gates and Special Gates | 30th Nov |  |
| 4. | VHDL Code for Adder | 7th Dec |  |
| 5. | VHDL Code for Subtractor | 14th Dec |  |
| 6. | VHDL Code for Multiplexer | 21st Dec |  |
| 7. | VHDL Code for De-Multiplexer | 4th Jan, 2026 |  |
| 8. | VHDL Code for Decoder and Encoder | 18th Jan |  |
| 9. | VHDL Code for Flip Flop | 25th Jan |  |

LAB 9: VHDL CODE FOR FLIP FLOP

## Theory

A flip-flop is a basic sequential logic circuit capable of storing one bit of binary information. Unlike combinational circuits, flip-flops have memory, meaning their output depends on the present input and the previous state. Flip-flops are triggered by a clock signal and are widely used in registers, counters, and memory elements.

## SR (Set-Reset) Flip-Flop

### Theory

The SR flip-flop has two inputs: S (Set) and R (Reset), along with outputs Q and Q̅.

- When S = 1, the output is set to 1

- When R = 1, the output is reset to 0

- When both inputs are 0, the output remains unchanged

- When both inputs are 1, the condition is invalid

### Truth Table (SR Flip-Flop)

| S | R | Q (Next State) |
| --- | --- | --- |
| 0 | 0 | No change |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Invalid |

### Limitation

The invalid condition (S = R = 1) is a major drawback of the SR flip-flop.

## JK Flip-Flop

### Theory

The JK flip-flop is an improved version of the SR flip-flop that eliminates the invalid state. It has two inputs: J and K.

- When both inputs are 0, the output remains unchanged

- When J = 1 and K = 0, the output is set

- When J = 0 and K = 1, the output is reset

- When J = K = 1, the output toggles

### Truth Table (JK Flip-Flop)

| J | K | Q (Next State) |
| --- | --- | --- |
| 0 | 0 | No change |
| 0 | 1 | 0 |
| 1 | 0 | 1 |
| 1 | 1 | Toggle |

### Advantage

The JK flip-flop has no invalid condition and is widely used in counters and registers.

## D (Data or Delay) Flip-Flop

### Theory

The D flip-flop has a single input D and eliminates ambiguity by ensuring that the output follows the input at the clock edge.

- The output Q takes the value of D when the clock is triggered

- It is commonly used for data storage and registers

### Truth Table (D Flip-Flop)

| D | Q (Next State) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

### Advantage

Simple operation and no invalid state.

## T (Toggle) Flip-Flop

### Theory

The T flip-flop is obtained from a JK flip-flop by tying J and K together.

- When T = 0, the output remains unchanged

- When T = 1, the output toggles
It is mainly used in binary counters.

### Truth Table (T Flip-Flop)

| T | Q (Next State) |
| --- | --- |
| 0 | No change |
| 1 | Toggle |

## Master-Slave Flip-Flop

### Theory

A Master-Slave flip-flop consists of two flip-flops connected in series to eliminate race-around condition.

- The master responds during the first half of the clock

- The slave responds during the second half
This ensures stable output changes.

## Comparison of Flip-Flops

| Flip-Flop | Inputs | Special Feature |
| --- | --- | --- |
| SR | S, R | Simple but has invalid state |
| JK | J, K | No invalid state |
| D | D | Output follows input |
| T | T | Toggles output |
| Master-Slave | Varies | Eliminates race condition |

## Applications of Flip-Flops

- Registers

- Counters

- Memory units

- Shift registers

- Timing and control circuits

SR FLIP FLOP

BEHAVIORAL:

library IEEE;

use IEEE.std_logic_1164.all;

entity arp_flipflop is

port(

S,R, Clk : in Std_logic;

Q, Qn : out Std_logic

);

end arp_flipflop;

architecture arch_arp of arp_flipflop is

signal preQ : Std_logic := '0';

begin

process(Clk, S, R)

begin

if (Clk = '1') then

if(S='0' and R='0') then

preQ <= preQ;

elsif (S='0' and R='1') then

preQ <= '0';

elsif (S='1' and R='0') then

preQ <= '1';

else

preQ <= 'X';

end if;

end if;

end process;

Q <= preQ;

Qn <= not preQ;

end arch_arp;

TESTBENCH:

library IEEE;

use IEEE.std_logic_1164.all;

entity test_flipflop is

end test_flipflop;

architecture test_flipflop_arch of test_flipflop is

component arp_flipflop is

port(

S,R, Clk : in Std_logic;

Q, Qn : out Std_logic

);

end component;

signal S, R, Clk : Std_logic;

signal Q, Qn : Std_logic;

begin

UUT: arp_flipflop port map

(

S => S,

R => R,

Clk => Clk,

Q => Q,

Qn => Qn

);

clk_process : process

begin

Clk <= '0';

wait for 0 ps;

Clk <= '1';

wait for 5 ps;

end process;

stim_proc : process

begin

S <= '0'; R <= '0';

wait for 100 ps;

S <= '0'; R <= '1';

wait for 100 ps;

S <= '1'; R <= '0';

wait for 100 ps;

S <= '1'; R <= '1';

wait for 100 ps;

end process;

end test_flipflop_arch;

WAVEFORM:

JK FLIP FLOP

BEHAVIORAL:

library IEEE;

use IEEE.std_logic_1164.all;

entity arp_flipflop is

port(

S,R, Clk : in Std_logic;

Q, Qn : out Std_logic

);

end arp_flipflop;

architecture arch_arp of arp_flipflop is

signal preQ : Std_logic := '0';

begin

process(Clk, S, R)

begin

if (Clk = '1') then

if(S='0' and R='0') then

preQ <= preQ;

elsif (S='0' and R='1') then

preQ <= '0';

elsif (S='1' and R='0') then

preQ <= '1';

else

preQ <= not preQ;

end if;

end if;

end process;

Q <= preQ;

Qn <= not preQ;

end arch_arp;

TESTBENCH:

library IEEE;

use IEEE.std_logic_1164.all;

entity test_flipflop is

end test_flipflop;

architecture test_flipflop_arch of test_flipflop is

component arp_flipflop is

port(

S,R, Clk : in Std_logic;

Q, Qn : out Std_logic

);

end component;

signal S, R, Clk : Std_logic;

signal Q, Qn : Std_logic;

begin

UUT: arp_flipflop port map

(

S => S,

R => R,

Clk => Clk,

Q => Q,

Qn => Qn

);

clk_process : process

begin

Clk <= '0';

wait for 0 ps;

Clk <= '1';

wait for 5 ps;

end process;

stim_proc : process

begin

S <= '0'; R <= '0';

wait for 100 ps;

S <= '0'; R <= '1';

wait for 100 ps;

S <= '1'; R <= '0';

wait for 100 ps;

S <= '1'; R <= '1';

wait for 100 ps;

end process;

end test_flipflop_arch;

# D FLIP-FLOP

## BEHAVIORAL

library IEEE;

use IEEE.std_logic_1164.all;

entity arp_dflipflop is

port(

D, Clk : in std_logic;

Q, Qn  : out std_logic

);

end arp_dflipflop;

architecture arch_arp of arp_dflipflop is

signal preQ : std_logic := '0';

begin

process(Clk, D)

begin

if (Clk = '1') then

preQ <= D;

end if;

end process;

Q  <= preQ;

Qn <= not preQ;

end arch_arp;

## TESTBENCH

library IEEE;

use IEEE.std_logic_1164.all;

entity test_dflipflop is

end test_dflipflop;

architecture test_arch of test_dflipflop is

component arp_dflipflop

port(

D, Clk : in std_logic;

Q, Qn  : out std_logic

);

end component;

signal D, Clk : std_logic;

signal Q, Qn  : std_logic;

begin

UUT: arp_dflipflop port map (D => D, Clk => Clk, Q => Q, Qn => Qn);

clk_process : process

begin

Clk <= '0'; wait for 5 ps;

Clk <= '1'; wait for 5 ps;

end process;

stim_proc : process

begin

D <= '0'; wait for 100 ps;

D <= '1'; wait for 100 ps;

D <= '0'; wait for 100 ps;

end process;

end test_arch;

# T FLIP-FLOP

## BEHAVIORAL

library IEEE;

use IEEE.std_logic_1164.all;

entity arp_tflipflop is

port(

T, Clk : in std_logic;

Q, Qn  : out std_logic

);

end arp_tflipflop;

architecture arch_arp of arp_tflipflop is

signal preQ : std_logic := '0';

begin

process(Clk, T)

begin

if (Clk = '1') then

if (T = '1') then

preQ <= not preQ;

else

preQ <= preQ;

end if;

end if;

end process;

Q  <= preQ;

Qn <= not preQ;

end arch_arp;

## TESTBENCH

library IEEE;

use IEEE.std_logic_1164.all;

entity test_tflipflop is

end test_tflipflop;

architecture test_arch of test_tflipflop is

component arp_tflipflop

port(

T, Clk : in std_logic;

Q, Qn  : out std_logic

);

end component;

signal T, Clk : std_logic;

signal Q, Qn  : std_logic;

begin

UUT: arp_tflipflop port map (T => T, Clk => Clk, Q => Q, Qn => Qn);

clk_process : process

begin

Clk <= '0'; wait for 5 ps;

Clk <= '1'; wait for 5 ps;

end process;

stim_proc : process

begin

T <= '0'; wait for 100 ps;

T <= '1'; wait for 100 ps;

T <= '1'; wait for 100 ps;

T <= '0'; wait for 100 ps;

end process;

end test_arch;

## Result

The SR, JK, D, and T flip-flops were successfully designed and simulated using VHDL behavioral modeling. The output waveforms obtained from the simulations matched the expected truth tables. Each flip-flop responded correctly to the clock signal, and the stored output changed according to the applied input conditions.

## Conclusion

This experiment successfully demonstrated the design and operation of basic sequential circuits using VHDL. The behavior of different flip-flops and their role in data storage and state control were clearly understood. The experiment confirmed that VHDL is an effective tool for modeling, simulating, and verifying flip-flop based digital circuits.