| Nepal College of Information Technology 2080 August | Examination; First Internal Assessment. Level; BE; Full Marks; 50. Programme; BCT; Pass Marks; 20. Year/Part; I/II; Time; 1.5 hrs. |
| --- | --- |

Subject: Instrumentation

√ Candidates are required to give their answers in their own words as far as practicable.

√ Attempt any 50 marks questions.

√ Assume suitable data if necessary.

- What are the basic block diagram of generalized instrumentation system? Draw the various blocks and explain their function.

- A multimeter having a sensitivity of 2,000 Ω/V is used for the measurement of voltage across a circuit having an output resistance of 10 kΩ. The open circuit voltage of the circuit is 6 V. Find the reading of the multimeter when it is set to its 10 V scale.  Find percentage error.

- The arms of a four arm bridge abcd, supplied with sinusoidal voltage, have the following values:

arm ab: A resistance of 200 Ω in parallel with a capacitance 1 µF.

arm bc: 400 Ω resistance, 	arm cd : 1000 Ω resistance.

arm da : A resistance R2 in series with a 2 µF capacitance.

Determine the value of R2 and the frequency at which the bridge will balance.

- A capacitive transducer uses two quartz diaphragms of area 750 mm2 separated by a distance of 3.5 mm. A pressure of 900 kN/m2 when applied to the top diaphragm produces a deflection of 0.6 mm. The capacitance is 370 pF when no pressure is applied to the diaphragms. Find the value of capacitance after the application of a pressure of 900 kN/m2.

- Define gauge factor. Derive an expression for gauge factor of a strain gauge.

- “Error of a potentiometric device is always negative except at two extreme points.” Justify it with necessary derivations.

- Define piezoelectric effect. Derive an expression for output voltage when a stress is applied to a piezoelectric transducer.

- Define isolation amplifier. State the importance of signal conditioning in instrumentation.

- What are the various type of data transmission medium? Explain Synchronous and Asynchronous data transmission in brief.

- Which is the fastest ADC? Explain successive approximation technique for A/D signal conversion.

- Convert an analog signal having the magnitude 3.625 in a 4-bit digital word successive approximation ADC if the reference voltage is 8V.

- Explain the principle of operation of Magnetic Tape Recorder. List out its areas of application

- Define data acquisition? Explain any one modern trend used for data acquisition.